| Jaguar F-PACE SVR 5.0-litre V8 Supercharged |  |
| --- | --- |
| ENGINE & TRANSMISSION |  |
| Engine capacity (cc) | 5,000 |
| Cylinders | 8 |
| Valves per cylinder | 4 |
| Bore/ stroke (mm) | 92.5/ 93 |
| Compression ratio | 9.5:1 |
| Fuel injection | Central spray guided Direct Injection |
| Boosting system | Supercharged |
| Power PS (kW) | 550 (405) |
| Torque Nm (lb ft) | 680 (502) |
| Transmission | ZF 8HP70 8-speed automatic |
| Gear ratios (:1) |  |
| 1st | 4.714 |
| 2nd | 3.143 |
| 3rd | 2.106 |
| 4th | 1.667 |
| 5th | 1.285 |
| 6th | 1.000 |
| 7th | 0.839 |
| 8th | 0.667 |
| Reverse | 3.317 |
| Final Drive | 3.230 |
| CHASSIS |  |
| Front suspension | Double wishbone |
| Rear suspension | Integral Link |
| Steering | Variable ratio rack-and-pinion; electromechanical |
| Front Brakes | Four-piston fixed caliper; 395mm two-piece disc |
| Rear Brakes | Single-piston sliding caliper; 396mm two-piece disc |
| DIMENSIONS & WEIGHTS |  |
| Length exc. number plate (mm) | 4,737 |
| Width inc./ folded mirrors (mm) | 2,175/ 2,071 |
| Height (mm) | 1,670 |
| Wheelbase (mm) | 2,874 |
| Track front/ rear (mm) | 1,648/ 1,666 |
| Unladen Weight (EU) (kg) | 2,070 |
| Unladen Weight (DIN) (kg) | 1,995 |
| Maximum towable mass (kg) | 2,400 |
| Maximum towable mass; braked/ unbraked trailers (kg) | 2,400/ 750 |
| Boot volume (litres) | 650 † |
| Fuel tank; usable (litres) | 82 |
| PERFORMANCE & FUEL ECONOMY |  |
| 0-60mph (sec) | 4.1 |
| 0-100km/h (sec) | 4.3 |
| Top speed mph (km/h) | 176 (283) |
| Fuel consumption mpg (litres/100km) EU combined | To be certified* |
| CO2 emissions (g/km) EU combined | To be certified* |

† With tyre repair system

*certification due in July 2018